Physics 2 Problem Solving – Capacitors in series, parallel and combination

- Three capacitors are placed in parallel to a battery as shown and  the capacitors are charged.

C1

V

C2

C3

C1 = 6 μF

C2 = 2 μF

C3 = 4 μF

V = 24 V

- Determine the equivalent capacitance.

Answer:  12 μF

- Determine the total charge on all capacitors.

Answer:  2.88 x 10-4 C

- Determine the volage across each capacitor.

Answer:  V1 = V2 = V3 = 24 V

- Determine the charge on each capacitor.

Answer: q1 = 1.44 x 10-4 C, q2 = 4.8 x 10-5 C, q3 = 9.6 x 10-5 C

- Three capacitors are placed in series to a battery as shown and  the capacitors are charged.

C1 = 6 μF

C2 = 2 μF

C1

V

C2

C3

C3 = 4 μF

V = 24 V

- Determine the equivalent capacitance.

Answer:  1.09 μF

- Determine the total charge on all capacitors.

Answer:  2.62 x 10-5 C

- Determine the volage across each capacitor.

Answer:  V1 = 4.37 V, V2 = 13.1 V.  V3 = 6.55 V

- Determine the charge on each capacitor.

Answer: q1 = q2 = q3 = 2.62 x 10-5 C

- Three capacitors are wire as shown below.

C1

V

C2

C3

C1 = 6 μF

C2 = 2 μF

C3 = 4 μF

V = 24 V

- Determine the equivalent capacitance.

Answer:  3.43 μF

- Determine the total charge on all capacitors.

Answer:  8.23 x 10-5 C

- Determine the voltage across C1.

Answer:  V1 = 13.7 V

- Determine the voltage across C2.

Answer:  V2 = V3 = 10.3 V